ALE WebSocket Configuration – Step by Step

Step 1: Download WebSocket server java from Aruba & setup then run WebSocket tunnelling server.

WebSocket is configured & running by default on Port 4443 with protocol ‘wss’ i.e. wss://0.0.0.0:4443. We can run Web/API requests on http://localhost:8700 or http://<hostname>:8700 to find out details about the configuration of WebSocket.

Find WebSocket server started details as below: (Highlighted required).

Step 2:

To Connect ALE server to WebSocket Tunnel we need to give below details:

- Local Endpoints

- Remote Endpoints

Step 3: Observe WebSocket tunnel server to reflect these changes, now the default 7779 (ZeroMQ port) & 8080 (REST API Port) port will be connected to WebSocket tunnel port i.e. 12000 & 12001

Step 4: Now to open above Web UI of Aruba ALE data can be retrieved from API i.e. http://localhost:8700, it is running when we started WebSocket tunnel server.

Run below command to check all above configured details:

curl http://127.0.0.1:8700/clients

Step 5: Now run Feed reader on this WebSocket tunnel port instead of direct port as below & observe that you will be retrieving same data s earlier.

feed-reader -e tcp://127.0.0.1:12000